| Product Name | EasyN IR Cut Nightvision CMOS Wireless WiFi IP Camera |  |
| --- | --- | --- |
| Brand Name | EasyN |  |
| Model No. | FS-613B-M166I |  |
| Features | System Security | Supports there-level account, password, user authority management |
|  | DDNS (free) | Free DDNS bounded? i.e., no need dyndns, like http://demo.easyn.hk. "demo" is the body code |
|  | IE Multi-Channel View Client | No need to install software, IE multi-channel?monitor, managment, message setting, save alarm picture, etc. |
|  | Super client (free) | 1, 4, 9, 16, 25, 36, 64-channel mutile software, no user limited professional software, centralized monitor, remote record and playback, Skype message alarm, skype telephone alarm |
|  | Superiority | support computer monitor, multi-channel view via iPhone |
| CPU | Operation System | Embedded Linux OS |
|  | Microcomputer Processor | 32Bit RSIC Embedded Processor |
| Video | Compression Format | Motion-JPEG-N |
|  | Signal System | CMOS 300,000 pixel |
|  | Frame Rate | 25fps |
|  | Resolution | VGA (640*480), QVGA (320*240) |
|  | Video Adjustment | brightness, contrast, chroma, saturation |
|  | Minimum Illumination | 0.1 Lux/F1.2 |
|  | WB. BLC | Auto |
|  | Lens | Standard: 3.6mm |
| Audio | Compression Format | ADPCM |
|  | Input | Built-in Mircophone |
|  | Output | Exteral audio interface |
|  | Audio Gain | AGC |
| Nightvision | Nightvision | 9 Φ5 LED; IR distance: 10m |
|  | Filter Switch | Built-in IR Cut, filter switch automatically, without color cast |
| P/T | Motor | Built-in motor control |
|  | Preset | Support 15 preset positions |
|  | Rotation Angle | Horizontal: 270 degrees; vertical: 90 degrees |
|  | Rotation Speed | 11 level speed manual speed range 15-70 degrees/second |
| Network | Network Interface | RJ-45 10/100M self-adaptable Ethernet slot |
|  | Protocol | Support TCP/IP, HTTP, ICMP, DHCP, FTP, SMTP, PPPoE, etc. |
|  | Online Visitor | Support 4 visitors at the same time; without limitation when using P2P |
|  | Wireless | WIFI, 802.11 b/g |
|  | IP Mode | Dynamic IP address, static IP address, PPPoE |
| Alarm | Input/Output | 1/1 |
|  | Motion Detection | Motion detection setting, sensitivity setting |
|  | Alarm Action | Support email, message alarm, ftp photo upload, preset positioncall , GPIO, etc. |
| System | System Requirement | Microsoft Win98/SE/ME/2000/XP/Vista/win7, Internet Explorer 8.0, or Firefox, Google browser |
| Certification | Certificate | ISO，FCC, CE, SASO |
| Others | Shape | Cartoon Robot |
|  | Power | DC 5V 2A |
|  | Use Environment | Indoor Use |
|  | Temperature | -10~50 Celsius Degrees |
|  | Humidity | 95% RH |
|  | Color | Brown (as shown in the pictures) |
|  | Size | Item size: 127 x 122 x 100mm |
|  |  | Package size: 170*170*170mm |
|  | Package Weight | 637g |
| Accessories | 1 * Power Adaptor (TYPE A plug) |  |
|  | 1 * Mounting Bracket |  |
|  | 1 * WIFI Antenna |  |
|  | 1 * Software |  |
|  | 1 * User manual |  |